| SAN DIEGO AIR POLLUTION CONTROL DISTRICT COMPLIANCE DIVISION 10124 OLD GROVE ROAD SAN DIEGO CA 92131 PHONE (858) 586-2650 | APCD USE ONLY |
| --- | --- |
|  | SECTOR |
|  | ID# |
|  | NOV# |

# VEEDER-ROOT VAPOR POLISHER HYDROCARBON EMISSIONS VERIFICATION TEST PROCEDURE

## Exhibit 12 of ARB E.O. VR 203-X and VR-204-X

Facility Name:	 A/C or PO Number:	 Date/Time of Test:	_

(Record exact time of test in order to demonstrate proper test sequencing as required in Attachment L)

| FACILITY AND TEST EQUIPMENT INFORMATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Vapor Polisher Percent Load1: | % |  |  |  |
| Is the Percent Load Greater than 80%? |  | ☐ YES | ☐ NO |  |
| Calibration Date of Flow Meter: |  |  |  |  |
| Certification/Re-Certification Date of Calibration Check Gas (<2 years): |  |  |  |  |
| Expiration Date of Inlet Test Gas: |  |  |  |  |
| Calibration & Inlet Gas Documentation Attached to this Form2? |  | ☐ YES | ☐ NO |  |
| CONVERSION: 9,000 ppm = 0.9% by volume = 50% LEL | Initial Check |  | After Re-Calibration (If Applicable ) ☐ N/A |  |
| HC Analyzer Zero Check Reading3: | ppm |  | ppm |  |
| Is the Zero Check Reading Less than 1,000 ppm (0.1% by Volume)? | ☐ YES | ☐ NO | ☐ YES | ☐ NO |
| HC Analyzer Calibration Check Reading3: | ppm |  | ppm |  |
| Is the Calibration Check Reading Between 8,000 and 10,000 ppm? | ☐ YES | ☐ NO | ☐ YES | ☐ NO |

| TEST RESULTS |  |  |
| --- | --- | --- |
| Vapor Control Valve Manually Opened?4 | ☐ YES | ☐ NO |
| 3-Way Ball Valve in Correct Testing Position? (Refer to Figure 1) | ☐ YES | ☐ NO |
| Start Flow Rate5 | SCFH |  |
| End Flow Rate5 | SCFH |  |
| HC Concentration after 3 minutes6 | ppm |  |
| HC Concentration after 4 minutes6 | ppm |  |
| HC Concentration after 5 minutes6 | ppm |  |
| HC Concentration after 6 minutes6 | ppm |  |
| HC Concentration less than 9,000 ppm throughout the entire test? | ☐ YES | ☐ NO |
| Vapor Control Valve returned to Automatic Mode? | ☐ YES | ☐ NO |
| 3-Way Ball valve returned to Normal Operating Position? | ☐ YES | ☐ NO |

1 Attach the Report used to determine the Vapor Polisher Percent Load to this test result form.

2 Calibration and inlet gas information as specified in Section 5.2 of Exhibit 12 shall be attached to this form.

3 HC Analyzer readings shall be recorded in parts per million (ppm).

4 Attach the IV800 RS232 Command Report to this test result form indicating the date and time the valve was manually opened.

5 The flow rates shall be recorded in standard cubic feet per hour (SCFH) and rounded to the nearest tenth (e.g. 0.1 SCFH).

6 Record the Hydrocarbon (HC) Concentration in parts per million (e.g. 1,000 ppm). All results less than 9,000 ppm shall be recorded as “<9,000 ppm”. All results greater than 9,000 ppm shall be recorded as “>9,000 ppm”.